Project Report: Pipelined vs Non-Pipelined CPU Simulator

1. Introduction

This project implements a comprehensive CPU instruction execution simulator that comparespipelinedandnon-pipelinedCPU architectures. The system consists of two components:

- GUI Application(C++/Win32) for visual simulation and comparison

- CLI Application(Assembly/emu8086) for command-line demonstration

The simulator allows users to input a sequence of simplified assembly-like instructions (ADD, SUB, LOAD, STORE, MUL, JMP) and calculates the number of clock cycles required under both execution models.

2. CPU Architecture Concepts

2.1 Non-Pipelined CPU

Anon-pipelined(or sequential) CPU executes instructions one at a time, completing all stages of an instruction before starting the next.

Characteristics:

- Simple design and implementation

- No instruction overlap

- Poor resource utilization

- Predictable but slow execution

Execution Pattern:

Instruction 1: [IF][ID][EX][MEM][WB]

Instruction 2: [IF][ID][EX][MEM][WB]

Instruction 3: [IF][ID][EX][MEM][WB]

2.2 Pipelined CPU

ApipelinedCPU overlaps instruction execution by dividing the instruction cycle into independent stages, allowing multiple instructions to be processed simultaneously.

Characteristics:

- Complex design with hazard detection

- High throughput (1 instruction/cycle in steady state)

- Better resource utilization

- Potential for pipeline hazards

5-Stage Pipeline:

Cycle 1: I1[IF]

Cycle 2: I1[ID] | I2[IF]

Cycle 3: I1[EX] | I2[ID] | I3[IF]

Cycle 4: I1[MEM] | I2[EX] | I3[ID] | I4[IF]

Cycle 5: I1[WB] | I2[MEM] | I3[EX] | I4[ID] | I5[IF]

2.3 Pipeline Stages

- IF (Instruction Fetch): Fetch next instruction from memory

- ID (Instruction Decode): Decode instruction and read registers

- EX (Execute): Perform ALU operation or address calculation

- MEM (Memory Access): Read/write data memory

- WB (Write Back): Write result to register file

3. Mathematical Formulas

3.1 Non-Pipelined Cycles Calculation

Non-Pipelined Cycles = (Number of Instructions) × (Number of Stages)

Example:5 instructions × 5 stages = 25 cycles

3.2 Pipelined Cycles Calculation

Pipelined Cycles = (Number of Stages) + (Number of Instructions - 1)

Example:5 stages + (5 instructions - 1) = 9 cycles

3.3 Speedup Calculation

Speedup = Non-Pipelined Cycles ÷ Pipelined Cycles

Example:25 ÷ 9 = 2.78x faster

3.4 Efficiency Metrics

- Throughput: Instructions per cycle (IPC)

- CPI: Cycles Per Instruction

- Efficiency Gain: Percentage improvement

4. GUI Application (C++/Win32)

4.1 Development Environment

- IDE: Eclipse CDT with MinGW/GCC

- Language: C++ with Win32 API

- Platform: Windows

4.2 Key Features

- Instruction Input: Support for 6 instruction types with validation

- Visual Simulation: Graphical representation of execution patterns

- Performance Comparison: Side-by-side cycle comparison

- Color-coded Interface: Professional UI with color scheme

- Detailed Analysis: Stage-by-stage breakdown

4.3 Workflow

1. User enters instruction (ADD/SUB/LOAD/STORE/MUL/JMP)

2. Click "Add" to add to instruction sequence

3. Instruction appears in the list

4. Click simulation buttons:

- "Non-Pipeline": Shows sequential execution analysis

- "Pipeline": Shows parallel execution with timeline

- "Compare": Shows performance comparison

- "Stages Info": Shows pipeline stage details

- Results display cycle counts, speedup, and efficiency

4.4 Class Structure

class PipelineSimulator {

private:

vector<string> instructions;

vector<Instruction> instrList;

int pipelineStages;

public:

// Core methods

void addInstruction(string instr);

int getNonPipelinedCycles();

int getPipelinedCycles();

float getSpeedup();

};

4.5 Color Scheme

- Background: Light blue (RGB 240, 245, 255)

- Text Areas: Ivory (RGB 255, 255, 240)

- Buttons: Steel blue (RGB 70, 130, 180)

- Lists: Light yellow (RGB 255, 250, 240)

- Titles: Dark blue (RGB 0, 0, 100)

5. CLI Application (Assembly/emu8086)

5.1 Development Environment

- Emulator: emu8086

- Language: x86 Assembly (16-bit)

5.2 Key Features

- Simple Interface: Command-line input/output

- Cycle Calculation: Basic pipeline vs non-pipeline comparison

- Instruction Validation: Checks for valid instructions

- Result Display: Clear formatting of results

- Educational: Demonstrates assembly programming concepts

5.3 Workflow

1. Program displays title and instructions

2. User enters instructions one by one

3. Type "END" to finish input

4. Program calculates:

- Non-pipelined cycles

- Pipelined cycles

- Speedup factor

- Cycles saved

5. Displays detailed analysis and pipeline information

6. Waits for key press before exiting

5.4 Assembly Implementation Details

; Key variables

inst_count DB 0 ; Instruction counter

np_cycles DW 0 ; Non-pipelined cycles

p_cycles DW 0 ; Pipelined cycles

; Main calculation functions

calculate_np: ; inst_count × 5

calculate_p: ; 5 +(instcount−1)calculate_speedup: ; np_cycles ÷ p_cycles

6. Comparison of Both Implementations

| Aspect | GUI Application | CLI Application |
| --- | --- | --- |
| Language | C++ with Win32 API | x86 Assembly |
| Platform | Windows with GUI | DOS/emu8086 |
| Input Method | Graphical input boxes | Command-line text |
| Output Format | Rich text with colors | Plain text |
| Visualization | Graphical timeline | Text-based analysis |
| Complexity | High (OOP, GUI) | Low (procedural) |
| Educational Value | Demonstrates modern GUI programming | Demonstrates low-level CPU concepts |

7. Instruction Types and Timing

| Instruction | Stages | Duration | Description |
| --- | --- | --- | --- |
| ADD/SUB | 5 | 1 cycle/stage | Arithmetic operations |
| LOAD/STORE | 5 | 2 cycles/stage | Memory access operations |
| MUL | 5 | 3 cycles/stage | Multiplication operation |
| JMP | 3 | 1 cycle/stage | Branch instruction |

8. Results and Analysis Example

Sample Input:

ADD, SUB, LOAD, STORE

Calculations:

Non-Pipelined: 4 × 5 = 20 cycles

Pipelined: 5 + (4-1) = 8 cycles

Speedup: 20 ÷ 8 = 2.5x faster

Cycles Saved: 20 - 8 = 12 cycles

Efficiency Gain: (12 ÷ 20) × 100 = 60%

Visual Timeline (Pipeline):

Cycle 1: I1[IF]

Cycle 2: I1[ID] | I2[IF]

Cycle 3: I1[EX] | I2[ID] | I3[IF]

Cycle 4: I1[MEM] | I2[EX] | I3[ID] | I4[IF]

Cycle 5: I1[WB] | I2[MEM] | I3[EX] | I4[ID]

Cycle 6: I2[WB] | I3[MEM] | I4[EX]

Cycle 7: I3[WB] | I4[MEM]

Cycle 8: I4[WB]